| Scheme of Valuation/Answer Key (Scheme of evaluation (marks in brackets) and answers of problems/key) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| APJ ABDUL KALAM TECHNOLOGICAL UNIVERSITY V SEMESTER B.TECH (S) DEGREE EXAMINATION, MAY 2019 |  |  |  |  |  |
| Course Code: CE305 |  |  |  |  |  |
| Course Name: GEOTECHNICAL ENGINEERING - II |  |  |  |  |  |
| Max. Marks: 100 |  |  |  | Duration: 3 Hours |  |
| PART A |  |  |  |  |  |
|  |  | Answer any two full questions, each carries 15 marks. |  |  | Marks |
| 1 | a) | Eqn. for vertical stress(σZ) beneath centre of circular footing --- 3 marks Calculation steps --- 1½ marks depth at which σZ reduces to 10% of the applied pressure= 1.854B ---3 marks |  |  | (7½ ) |
|  | b) | Kp1 & Kp2 = 3 & 5.828 ---- 1 mark Eqn. for passive pressure ---- 1 mark Calculation steps --- 1½ marks 4 values of passive pressure – 2 marks z (m); 0; 3-dz; 3+dz; 5. passive pressure (kPa); 0; 144; 279.8; 512.9. total passive earth pressure = 1008.7 kN/m ----2 marks | (7½ ) |
| 2 | a) | any 4 major limitations of Boussinesq’s theory ----6 marks reason for the theory to be still in use ---1½ marks |  |  | (7½ ) |
|  | b) | Ka = 0.26 ---- 1 mark Eqn. for active pressure ---- 1 mark Calculation steps --- 1½ marks 5 values of pressure [3due to soil; 2 due to water] – 2½ marks z (m); 0; 2; 5; z (m); 2; 5. active pressure exerted by the soil (kPa); 0; 8.31; 15.48; Lateral. Pressure exerted by water (kPa); 0.0; 29.43. total active earth pressure = 88.1kN/m ---1½ marks | (7½ ) |
| 3 | a) | Sketch showing rectangular footing & the areas to be considered – 3½ marks the (m,n) combinations to be used = (1,1.5) & (1,0.25) --- 4 marks |  |  | (7½ ) |
|  | b) | Any 6 major assumptions in Rankine’s theory --- 7½ marks. |  |  | (7½ ) |
| PART B |  |  |  |  |  |
| Answer any two full questions, each carries 15 marks. |  |  |  |  |  |
| 4 | a) | soil types for which LSF can be expected ---4 marks typical pressure versus settlement curve for LSF --- 3½ marks.. |  |  | (7½ ) |
|  | b) | Definition sketch – 2marks Calculation steps --- 1½ marks Distance to the C.G. of footing from axis of 850kN column = 2.1m --- 1 mark Ans: L=4.7m -- 1½ marks B=2m -------1½ marks |  |  | (7½ ) |
| 5 | a) | - Both are strip footings ------ 2 marks - Eqn. for net safe bearing capacity (qns)of strip footing ------ 2 mark s . Calculation steps --- 1½ marks - (qns)A/ (qns)B = 1.67 ------ 2 marks |  |  | (7½ ) |
|  | b) | - Mentioning any 3 methods for rectification of tilts in a well foundation---- 3 marks - 3 neat sketches------ 4½ marks |  |  | (7½ ) |
| 6 | a) | - Definition of Gross UBC, Net UBC, Net SBC ------ 6 marks definition of Allowable bearing capacity ----.1½ marks |  |  | (7½ ) |
|  | b) | Any 2 situations where raft foundations are preferred------ 4 marks concept of floating foundation------ 3½ marks |  |  | (7½ ) |
| PART C |  |  |  |  |  |
| Answer any two full questions, each carries20 marks. |  |  |  |  |  |
| 7 | a) | - Skin friction capacity of friction pile in sands - = K× tan δ × perimeter of pile × area of effective pressure diagram---- 3 marks - Effective overburden pressure diagram --- ---- 2 marks - Calculation steps --- 2 marks - ultimate load on the pile=370.9 kN---- 3marks. |  |  | (10 ) |
|  | b) | any 2 merits of auger boring method ---- 2 marks soil types for which the method is applicable---- 3 marks Mentioning any 2 types of augers ---- 2 marks Neat sketch of any one type of auger – 3 marks |  |  | (10 ) |
| 8 | a) | - UBC of a single friction pile = [α. c. As] ---- 2 marks - UBC of the Block = [c. As] --- 2 marks - Calculation steps --- 2 marks - UBC a single friction pile = 471.2 kN---- 1 mark - Length of pile block= 2.2m ---- 1marks - UBC of the Block = 4400 kN ---- 1 marks - UBC of the PILE GROUP = 4241kN ---- 1 marks |  |  | (10 ) |
|  | b) | Meaning of free vibration -------- 4 marks Brief discussion of any 3 methods for vibration isolation. ---- 6 marks |  |  | (10 ) |
| 9 | a) | Meaning of negative skin friction (Qnsf)---- 3 marks Sketch --- 2 marks (Qnsf) = [α. c. As] ---- 3 marks (Qnsf) = 29.45kN---- 2 marks |  |  | (10 ) |
|  | b) | - dilatancy correction –- 2 marks - soil types/soil states for which the correction is applied ( fine sands and silts; below WT; N˃15) ---- 6 marks -- - Eqn.for dilatancy correction --- 2 marks |  |  | (10) |
| **** |  |  |  |  |  |